Habasit AG, 
P.O. Box, CH-4153 Reinach-Basel
Phone 
++41 61 715 15 15
Fax
++41 61 715 15 55
Trouble shooting instructions

PQ-57, PQ-58
[image: image4.wmf]
[image: image5.wmf]
Author:
Ni
Page 1of 1 
Edition:
9902
Subject to alterations
Replaces:
-

[image: image2.wmf]
Trouble shooting instructions

PQ-57, PQ-58
[image: image3.wmf]


Page 2 of 1 
Edition:
9902
Subject to alterations

Betriebsanleitung 37001


Step-Flex Stanzapparat

AZ-1200




Seite 13 von 1 
Ausgabe:
9612
Änderungen vorbehalten


Trouble shooting instructions

The most common problems encountered when operating the PQ-57 and PQ-58 presses can be analyzed and fixed quite easily. Instructions on how to find the failed part and how to repair are given below.

WARNING
This tool is only to be serviced by a qualified electrician. Observe your local regulations!

1.
Finding and repairing the point of failure

Three elements in the PQ-57, PQ-58 may be replaced in the field after check of the press has found them to be defective:

· The heating cartridge (heater wire may burn with age)

· The thermofuse (burns, when press overheats, for whatever reason)

· The light indicator.

· Check the light indicator. If it does not light, but the press does heat, there is either a loose contact or the indicater itself is broken. 

INDICATION
After reaching nominal temperature, the light indicator only flashes for short intervals. Therefore best check function of the light indicator during heating up.

If the light indicator does not work: 

· open the press.

· check correct contact of the light indicator.

· Replace light indicator if necessary.

· Open the cover of both press handles and check resistance of the following elements: (Refer to the wiring diagrams in the operating instruction. Make sure that you have the version for the right voltage):

· Lower heater cartridge (RL01, 59 ( nominal).

· Upper heater catridge (RL02, 59 ( nominal).

· Thermofuse (TF1, 0 ( nominal).

· If either of these elements is broken (infinite resistance), replace it. 

· Replace heater cartridges also if they deviate more than 15 % from their nominal resistance.

2.
Re-assembly of the press

This is not a complete assembly instruction that defines each part that is required and where it has to go. It is assumed that the person doing this task has either disassembled the press before and knows the basic setup, or has an assembled press for reference.

The purpose of this section is to explain the non trivial task of re-assembling the spring loaded hinge of the tong.

2.1
Tools needed

Prepare a cylindrical pin, diameter 8mm with a conical end:

[image: image1.wmf]Æ

 7.5 ... 8 mm

Æ

 0.29 ... 0.31“

min 50 mm

min 2“

Cone, 5:1 ... 10:1


2.2
Assembly

· Wire electrical components according to the wiring diagram in the operating instructions.

· Re-assemble upper and lower press halves side by side, without covers.

· Slide the two springs on the hinge bush and insert the assembly into the lower press half. Make sure that the ends of the springs find their proper location and don’t catch any wires. The wiring between the two halves goes around the hinge bush in front.

· Flip the upper press half over and set it on top. Insert the spring ends in their proper location.

· Hold the upper press half horizontal, press it down and insert the conical pin through hinge holes and hinge bush.

· Insert the hinge pin proper, pushing out the conical pin in the process.

· Fasten E-clips an re-install covers.

WARNING
Don’t connect the press to mains before it is completely assembled and checked!
C:\TEMP\i990301a.doc printed 28.08.98 11:14
C:\TEMP\i990301a.doc printed 28.08.98 11:14
C:\TEMP\i990301a.doc printed 28.08.98 11:14 

[image: image2.wmf][image: image3.wmf][image: image4.wmf][image: image5.wmf]_965808083

_965808085

